- Introduction

1.1 Introduction to the Global Flooding Crisis:

Around the globe, increasing flood events have become a more pressing concern, underscored by the devastating consequences on human life, economies, and ecosystems. These trends are not merely acts of nature. They are exacerbated by human activities that disrupt natural and hydrological processes, such as landscape alterations resulting from deforestation, urbanization, and infrastructural development (Ceola et al., 2014). Compounding these challenges is climate change, as rising global temperatures lead to more intense rainfall events and increased flood likelihood (Frederikse et al., 2022). Further, the drive for growth and development has seen populations expand into flood-prone areas, elevating human exposure and vulnerability to these hazards (Ceola et al., 2014).

1.2 The Intersection of Floods and Urban Infrastructure:

It is not just homes and lives at risk. Urban infrastructure, notably roads and transportation networks bear the brunt of these flooding events. When floods strike, roads can experience anything from surface erosions to complete structural failures, depending on the severity and duration of the flood (Pant et al., 2017). This obstructs daily commutes, disrupts economic activities and critically hampers emergency response efforts. Access to essential services becomes strained, and evacuating residents from danger zones becomes challenging. The very backbone of urban connectivity comes under threat.

1.3 Towards Resilient Infrastructure:

Therefore, building resilient road networks is not just an infrastructural necessity—it is a lifeline. In this context, resilience is a system's ability to anticipate, adapt to, and swiftly recover from disturbances (Kong et al., 2019). In road networks, resilience entails the capacity to withstand and recover from flood events, minimizing disruptions to transportation services. Road networks need more than just robust construction; they require strategic design considerations like elevated sections, flood-resistant materials, and optimized drainage systems to be resilient (Li et al., 2019). Advanced forecasting can further this resilience, enabling proactive planning (Wang et al., 2019).

1.4 Leveraging Modern Predictive Techniques:

Traditional hydrological models have their merits, but they also have limitations when capturing the intricate dynamics of flood processes. Machine learning, with its prowess in modelling nonlinear relationships and yielding accurate forecasts, stands at the forefront of modern flood prediction efforts (Yang & Chang, 2020; Mosavi et al., 2018). Machine learning models, like ANNs and support vector machines, have been applied to tasks like runoff forecasting with notable success, illuminating patterns in historical data and providing timely, critical forecasts (Lu et al., 2022; Mosavi et al., 2018). By reducing the uncertainties in flood prediction, we can effectively mitigate property damages and, most importantly, the loss of human lives (Chen, 2021).

1.5 This Study's Focal Point - The 2018 Kerala Floods:

The urgency and significance of this research become even more palpable when considering real-world scenarios, such as the 2018 floods in Kerala, India. The state received rainfall that was off the charts—2346.6 mm from June to August—leading to severe flooding in 13 districts (Babu & Leno, 2020). This deluge brought about widespread destruction, touching every sector from agriculture to animal husbandry (V.L., 2021; Veerakumaran & Santhi, 2019). Most critically, for this study, road networks in Kerala faced extensive damages, with many regions rendered inaccessible, underscoring the interconnected devastation a flood can bring (Mishra et al., 2018; Senan et al., 2022).

1.6 Objective and Significance of the Research:

Given these challenges, this project aims to create an algorithm capable of predicting road disruptions during intense precipitation events. The project plans to achieve this through the following objectives:

- To assess the vulnerability of road networks to flooding, focusing on factors such as elevation, slope, permeability and distance to drainage.

- To design and fine-tune a machine learning-based algorithm that can accurately forecast the likelihood of road flooding during heavy precipitation events.

- To validate the algorithm using historical flood data, particularly from the 2018 floods in Kerala, ensuring its robustness and reliability in real-world scenarios.

Accurate predictions empower authorities and communities to plan and deploy resources efficiently. Also, as someone who has experienced the floods in this study personally, it is not just about maintaining transportation—it is about fortifying the resilience of an entire community against the destructive power of floods.

- Literature Review

  - Importance of Flood Prediction

The importance of flood prediction cannot be overstated, as it plays a crucial role in mitigating the adverse impacts of floods and improving disaster management strategies. Floods are among the most destructive natural disasters, causing extensive damage and loss of life (Mosavi et al., 2018). Therefore, accurate flood prediction models are crucial for risk reduction, policy development, and minimizing the loss of human life and property damage associated with floods (Mosavi et al., 2018). Flood prediction also plays a vital role in urban planning and infrastructure development. By accurately predicting flood events, city planners can identify areas at high risk of flooding and implement appropriate measures to mitigate the impact. This may include constructing flood barriers, improving drainage systems, or implementing land-use regulations to prevent construction in flood-prone areas (Danso et al., 2020). Accurate flood prediction can also inform the design and construction of critical infrastructure, such as bridges and dams, ensuring they are built to withstand potential flood events (Jafarzadegan et al., 2023). Furthermore, flood prediction models can help in the management of water resources. Water resource managers can make informed decisions about water allocation and reservoir operations by accurately predicting flood events. This can help prevent overflows and ensure the efficient use of water resources during flood events (Zhang et al., 2018).

  - Diverse Approaches to Flood Prediction

Several different flood prediction models are available that utilize various techniques and approaches. These models aim to accurately predict the occurrence and severity of floods to support decision-making and mitigate the impacts of flooding. Some of the notable flood prediction models include:

1. Machine Learning Models: Machine learning techniques, such as artificial neural networks (ANN), support vector regression (SVR), and random forest, have been widely used in flood prediction (Mosavi et al., 2018). These models utilize historical flood data and other relevant variables to train the model and make predictions. They have shown promising results in accuracy and performance (Esfandiari et al., 2020).

2. Hydrological Models: Hydrological models, such as the Hydrologic Engineering Center's Hydrologic Modeling System (HEC-HMS), use mathematical equations to simulate the hydrological processes that lead to flooding (Banihabib et al., 2020). These models predict flood events using rainfall, soil moisture, and river flow. They are commonly used for riverine flood prediction.

3. Statistical Models: Statistical models, such as the index flood method and quantile regression, estimate flood quantiles and predict flood events (Ishak et al., 2011). These models analyze historical flood data and identify patterns and relationships between flood variables. They are often used when at-site recorded flood data are limited.

4. Data-Driven Models: Data-driven models, such as Gaussian processes and deep learning models, utilize large datasets to make predictions (Brockkötter et al., 2021). These models can capture complex relationships and patterns in the data and provide accurate flood predictions (Qian, 2019). They have shown promising results in real-time flood forecasting and early warning systems (Wu et al., 2020).

5. Ensemble Models: Ensemble models combine multiple individual models to improve the accuracy and reliability of flood predictions (Choubin et al., 2019). These models take advantage of the strengths of different models and reduce the uncertainties associated with individual models (Berkhahn et al., 2019). Ensemble models can include a combination of machine learning, hydrological, and statistical models.

6. Remote Sensing Models: Remote sensing data, such as satellite imagery and radar data, can be used in flood prediction models (Schumann et al., 2009). These models utilize the information from remote sensing technologies to estimate flood extent and monitor flood dynamics (Mah et al., 2012). Remote sensing models can provide valuable input data for other flood prediction models.

7. Physics-Based Models: Physics-based models, such as the Saint Venant equations, simulate the physical processes of water flow and flooding (Elhanafy & Copeland, 2007). These models consider the fundamental principles of fluid dynamics and hydraulic behaviour to predict flood events (Lhomme et al., 2008). Physics-based models are often used for detailed flood simulations and floodplain inundation mapping.

2.3 Machine Learning in Flood Prediction

In the realm of flood prediction, various machine learning models have gained prominence. The Support Vector Machine (SVM) stands out for its versatility in handling both classification and regression, finding application in flood susceptibility mapping and forecasting (Li et al.,2019; Liu et al., 2021; Tehrany et al., 2015; Tehrany et al., 2014; Ha et al., 2021). The Random Forest (RF), amalgamates numerous decision trees, enhancing the accuracy of flood predictions (Khalaf et al., 2020; Sampurno et al., 2022; Granata et al., 2016; Li et al., 2016). Artificial Neural Networks (ANN), drawing inspiration from human neural structures, are recognized for their prowess in deciphering intricate nonlinear relationships (Atashi et al., 2022; Tayfur et al., 2018; Hu et al., 2021). Decision trees, with their decision-making tree-like frameworks, have been instrumental in flood forecasting (Choubin et al., 2019; Antonetti et al., 2019). Ensemble models, which merge algorithms like SVM, RF, and ANN, prioritize prediction precision (Jia et al., 2022; Rahman et al., 2021). The nonlinear regression approach, Multivariate Adaptive Regression Splines (MARS), is lauded for addressing complex relationships in flood dynamics (Costache & Bui, 2020). Deep learning models, encompassing Convolutional Neural Networks (CNN) and Recurrent Neural Networks (RNN), have been impactful given their capacity to process voluminous data and identify spatial-temporal flood patterns (Chen et al., 2023; Kondo et al., 2023). Lastly, hybrid models, which merge varied algorithms or couple machine learning with other models like hydrological ones, are tailored to harness the merits of diverse methodologies, aiming for enhanced flood prediction accuracy (Liu et al., 2021; Pham et al., 2017; Xie & Shen, 2021).

2.4 Predictive Features used in ML based Flood Models

In flood prediction model research, a plethora of features have been utilized to optimize their predictive capacities. Meteorological data, encompassing aspects like rainfall patterns and humidity, provides a foundational understanding of precipitation dynamics leading to floods (Ward et al., 2014; Chang et al., 2020). Complementing this, hydrological variables, such as river flow rates and discharge measurements, elucidate the riverine responses to precipitation events (Tamiru & Wagari, 2021; Tabarestani & Afzalimehr, 2021). The terrain's influence on flood dynamics is assessed through topographic data, highlighting elevation and slope gradient factors that direct water flow (Li et al., 2021; Wang et al., 2021). Land use and land cover data, which details the spatial arrangement of urban areas, forests, and other terrains, further influences flood genesis by affecting infiltration and runoff processes (Li et al., 2021). Soil properties, including moisture content and infiltration capacity, contribute to runoff generation dynamics (Wang et al., 2021; Tabarestani & Afzalimehr, 2021). Advancements in technology, notably remote sensing, offer insights on vegetation and surface water distribution, enhancing the spatial comprehension of flood variables (Munawar et al., 2022). Historical flood data, encompassing prior flood locations and characteristics, serves as an essential reference to discern patterns and relationships (Mistry & Parekh, 2022; Tabbussum & Dar, 2021). Lastly, other contingent factors such as anthropogenic influences and dam operations might be factored into models depending on the specific research context (Ward et al., 2014; Tabbussum & Dar, 2021).

2.5 Advantages of ML Based Models

Machine learning models present a range of superiorities in flood prediction compared to conventional techniques. Firstly, they excel in discerning complex relationships and pinpointing intricate patterns and nonlinear interactions, which standard models might often miss (Atashi et al., 2022). Anchored in a data-driven paradigm, they harness extensive historical flood records, enabling them to refine and enhance predictions iteratively. This adaptability extends to diverse flood types and geographic areas, and by tailoring them to specific datasets, they can be aptly aligned with a region's unique attributes (Kohansarbaz et al., 2022). An integrative capability allows these models to amalgamate data from diverse avenues like remote sensing, meteorological records, and past flood events (Sankaranarayanan et al., 2019). Addressing one of the recurrent challenges, these models capably manage missing or fragmentary data, ensuring consistent forecasting despite data gaps (Tayfur et al., 2018). Emphasizing their adaptability, they are scalable, proficiently handling extensive tasks, a feature pivotal for prominent urban localities or expansive river catchments (Kondo et al., 2023).

References

- Asim, M., Mekkodathil, A., Sathian, B., Elayedath, R., N, R. K., Simkhada, P., … & Teijlingen, E. v. (2019). Post-traumatic stress disorder among the flood affected population in indian subcontinent. Nepal Journal of Epidemiology, 9(1), 755-758. https://doi.org/10.3126/nje.v9i1.24003

- Antonetti, M., Horat, C., Sideris, I. V., & Zappa, M. (2019). Ensemble flood forecasting considering dominant runoff processes – part 1: set-up and application to nested basins (emme, switzerland). Natural Hazards and Earth System Sciences, 19(1), 19-40. https://doi.org/10.5194/nhess-19-19-2019

- Atashi, V., Gorji, H. T., Shahabi, S. M., Kardan, R., & Lim, Y. H. (2022). Water level forecasting using deep learning time-series analysis: a case study of red river of the north. Water, 14(12), 1971. https://doi.org/10.3390/w14121971

- Babu, S. M. and Leno, N. (2020). Organic carbon status in flood affected high hills of kerala. International Journal of Chemical Studies, 8(4), 3033-3035. https://doi.org/10.22271/chemi.2020.v8.i4ak.10111

- Bach, M., Werner, A., Żywiec, J., & Pluskiewicz, W. (2017). The study of under- and over-sampling methods’ utility in analysis of highly imbalanced data on osteoporosis. Information Sciences, 384, 174-190. https://doi.org/10.1016/j.ins.2016.09.038

- Banihabib, M. E., Jurík, Ľ., Kazemi, M. S., Soltani, J., & Tanhapour, M. (2020). A hybrid intelligence model for the prediction of the peak flow of debris floods. Water, 12(8), 2246. https://doi.org/10.3390/w12082246

- Berkhahn, S., Fuchs, L., & Neuweiler, I. (2019). An ensemble neural network model for real-time prediction of urban floods. Journal of Hydrology, 575, 743-754. https://doi.org/10.1016/j.jhydrol.2019.05.066

- Brockkötter, J., Ahndorf, J., & Jupke, A. (2021). Prediction of flooding in packed liquid‐liquid and high‐pressure extraction columns using a gaussian process. Chemie Ingenieur Technik, 93(12), 1907-1916. https://doi.org/10.1002/cite.202100073

- Ceola, S., Laio, F., & Montanari, A. (2014). Satellite nighttime lights reveal increasing human exposure to floods worldwide. Geophysical Research Letters, 41(20), 7184-7190. https://doi.org/10.1002/2014gl061859

- Chang, D., Yang, S., Hsieh, S. L., Wang, H. J., & Yeh, K. (2020). Artificial intelligence methodologies applied to prompt pluvial flood estimation and prediction. Water, 12(12), 3552. https://doi.org/10.3390/w12123552

- Rahman, M. M. and Davis, D. N. (2013). Addressing the class imbalance problem in medical datasets. International Journal of Machine Learning and Computing, 224-228. https://doi.org/10.7763/ijmlc.2013.v3.307

- Chen, Y., Zhang, X., Yang, K., Zeng, S., & Hong, A. (2023). Modeling rules of regional flash flood susceptibility prediction using different machine learning models. Frontiers in Earth Science, 11. https://doi.org/10.3389/feart.2023.1117004

- Choubin, B., Moradi, E., Golshan, M., Adamowski, J., Hosseini, F. S., & Mosavi, A. (2019). An ensemble prediction of flood susceptibility using multivariate discriminant analysis, classification and regression trees, and support vector machines. Science of the Total Environment, 651, 2087-2096. https://doi.org/10.1016/j.scitotenv.2018.10.064

- Costache, R. and Bui, D. T. (2020). Identification of areas prone to flash-flood phenomena using multiple-criteria decision-making, bivariate statistics, machine learning and their ensembles. Science of the Total Environment, 712, 136492. https://doi.org/10.1016/j.scitotenv.2019.136492

- Dang, A. T. N. and Kumar, L. (2017). Application of remote sensing and gis-based hydrological modelling for flood risk analysis: a case study of district 8, ho chi minh city, vietnam. Geomatics, Natural Hazards and Risk, 8(2), 1792-1811. https://doi.org/10.1080/19475705.2017.1388853

- Danso, S. Y., Yi, M., Adjakloe, Y. A., & Addo, I. Y. (2020). Application of an index-based approach in geospatial techniques for the mapping of flood hazard areas: a case of cape coast metropolis in ghana. Water, 12(12), 3483. https://doi.org/10.3390/w12123483

- Dhasmana, M. K., Mondal, A., & Zachariah, M. (2023). On the role of climate change in the 2018 flooding event in kerala. Environmental Research Letters, 18(8), 084016. https://doi.org/10.1088/1748-9326/ace6c0

- Divakaran, S. J., Philip, J. S., Chereddy, P., Nori, S. R. C., Ganesh, A. J., John, J., … & Nelson-Sathi, S. (2019). Insights into the bacterial profiles and resistome structures following the severe 2018 flood in kerala, south india. Microorganisms, 7(10), 474. https://doi.org/10.3390/microorganisms7100474

- Dixit, A., Sahany, S., Rajagopalan, B., & Choubey, S. (2022). Role of changing land use and land cover (lulc) on the 2018 megafloods over kerala, india. Climate Research, 89, 1-14. https://doi.org/10.3354/cr01701

- Elhanafy, H. and Copeland, G. J. M. (2007). Flash floods simulation using saint venant equations. International Conference on Aerospace Sciences and Aviation Technology, 12(ASAT CONFERENCE), 1-14. https://doi.org/10.21608/asat.2007.23886

- Esfandiari, M., Abdi, G., Jabari, S., McGrath, H., & Coleman, D. (2020). Flood hazard risk mapping using a pseudo supervised random forest. Remote Sensing, 12(19), 3206. https://doi.org/10.3390/rs12193206

- Ferencevic, M. V. and Ashmore, P. (2011). Creating and evaluating digital elevation model-based stream-power map as a stream assessment tool. River Research and Applications, 28(9), 1394-1416. https://doi.org/10.1002/rra.1523

- Frederikse, T., Lee, T., Wang, O., Kirtman, B., Becker, E., Hamlington, B. D., … & Waliser, D. E. (2022). A hybrid dynamical approach for seasonal prediction of sea‐level anomalies: a pilot study for charleston, south carolina. Journal of Geophysical Research: Oceans, 127(8). https://doi.org/10.1029/2021jc018137

- George, R., Joseph, A., & Rema, K. P. (2022). Application of hydrologic engineering centers hydrologic modeling system (hec-hms) model for modelling flood in sub basin of meenachil river, kerala, india. International Journal of Environment and Climate Change, 1251-1262. https://doi.org/10.9734/ijecc/2022/v12i121564

- Ginkel, K. v., Dottori, F., Alfieri, L., Feyen, L., & Koks, E. (2021). Flood risk assessment of the european road network. Natural Hazards and Earth System Sciences, 21(3), 1011-1027. https://doi.org/10.5194/nhess-21-1011-2021

- Granata, F., Gargano, R., & Marinis, G. d. (2016). Support vector regression for rainfall-runoff modeling in urban drainage: a comparison with the epa’s storm water management model. Water, 8(3), 69. https://doi.org/10.3390/w8030069

- Greeshma, S. and Greeshma, M. (2023). The economic impact of floods on the plantation sector: a study of selected districts in kerala. Disaster Advances, 16(3), 13-22. https://doi.org/10.25303/1603da013022

- Ha, H., Luu, C., Bui, Q. D., Pham, D., Hoang, T., Nguyen, V., … & Pham, B. T. (2021). Flash flood susceptibility prediction mapping for a road network using hybrid machine learning models. Natural Hazards, 109(1), 1247-1270. https://doi.org/10.1007/s11069-021-04877-5

- Hashemi-Beni, L., Jones, J., Thompson, G. L., Johnson, C., & Gebrehiwot, A. (2018). Challenges and opportunities for uav-based digital elevation model generation for flood-risk management: a case of princeville, north carolina. Sensors, 18(11), 3843. https://doi.org/10.3390/s18113843

- Hou, L., Wang, Y., Shen, F., Lei, M., Wang, X., Zhao, X., … & Alhaj, A. (2020). Study on variation of surface runoff and soil moisture content in the subgrade of permeable pavement structure. Advances in Civil Engineering, 2020, 1-12. https://doi.org/10.1155/2020/8836643

- Hu, Q., Zhu, Y., Hu, H., Guan, Z., Qian, Z., & Yang, A. (2021). Multiple kernel learning with maximum inundation extent from modis imagery for spatial prediction of flood susceptibility.. https://doi.org/10.21203/rs.3.rs-685721/v1

- Huang, M. and Jin, S. (2020). Rapid flood mapping and evaluation with a supervised classifier and change detection in shouguang using sentinel-1 sar and sentinel-2 optical data. Remote Sensing, 12(13), 2073. https://doi.org/10.3390/rs12132073

- Hunt, K. M. R. and Menon, A. (2020). The 2018 kerala floods: a climate change perspective. Climate Dynamics, 54(3-4), 2433-2446. https://doi.org/10.1007/s00382-020-05123-7

- Ishak, E. H., Haddad, K., Zaman, M. A., & Rahman, A. (2011). Scaling property of regional floods in new south wales australia. Natural Hazards, 58(3), 1155-1167. https://doi.org/10.1007/s11069-011-9719-6

- Jafarzadegan, K., Moradkhani, H., Pappenberger, F., Moftakhari, H., Bates, P., Abbaszadeh, P., … & Duan, Q. (2023). Recent advances and new frontiers in riverine and coastal flood modeling. Reviews of Geophysics, 61(2). https://doi.org/10.1029/2022rg000788

- Jia, J., Cui, W., & Liu, J. (2022). Urban catchment-scale blue-green-gray infrastructure classification with unmanned aerial vehicle images and machine learning algorithms. Frontiers in Environmental Science, 9. https://doi.org/10.3389/fenvs.2021.778598

- Jose, M. and Fenn, J. (2021). Post-traumatic stress disorder and resilience among flood affected farmers of kerala, india. International Journal of Community Medicine and Public Health, 8(4), 1842. https://doi.org/10.18203/2394-6040.ijcmph20211243

- Joseph, J. E. (2022). Social media response in disaster management with special reference to the kerala floods. Society and Culture Development in India, 2(2), 291-303. https://doi.org/10.47509/scdi.2022.v02i02.04

- Khalaf, M., Alaskar, H., Hussain, A. J., Baker, T., Maamar, Z., Buyya, R., … & Al‐Jumeily, D. (2020). Iot-enabled flood severity prediction via ensemble machine learning models. IEEE Access, 8, 70375-70386. https://doi.org/10.1109/access.2020.2986090

- Kohansarbaz, A., Shabanlou, S., Yosefvand, F., & Rajabi, A. (2022). Modelling flood susceptibility in northern iran: application of five well‐known machine‐learning models. Irrigation and Drainage, 71(5), 1332-1350. https://doi.org/10.1002/ird.2745

- Kondo, R., Du, B., Narusue, Y., & Morikawa, H. (2023). Machine learning framework supervised by hydraulic mechanical models for real-time pluvial flood prediction. Journal of Information Processing, 31(0), 256-264. https://doi.org/10.2197/ipsjjip.31.256

- Kong, J., Simonović́, S. P., & Zhang, C. (2019). Resilience assessment of interdependent infrastructure systems: a case study based on different response strategies. Sustainability, 11(23), 6552. https://doi.org/10.3390/su11236552

- Kumar, V., Pradhan, P. K., Sinha, T., Rao, S. V. B., & Chang, H. (2020). Interaction of a low-pressure system, an offshore trough, and mid-tropospheric dry air intrusion: the kerala flood of august 2018. Atmosphere, 11(7), 740. https://doi.org/10.3390/atmos11070740

- Lamb, R. A., Garside, P., Pant, R., & Hall, J. W. (2019). A probabilistic model of the economic risk to britain's railway network from bridge scour during floods. Risk Analysis, 39(11), 2457-2478. https://doi.org/10.1111/risa.13370

- Lhomme, J., Sayers, P., Gouldby, B., Samuels, P., Wills, M., & Mulet-Marti, J. (2008). Recent development and application of a rapid flood spreading method. Flood Risk Management: Research and Practice, 15-24. https://doi.org/10.1201/9780203883020.ch2

- Li, Z., Liu, H., Luo, C., & Fu, G. (2021). Assessing surface water flood risks in urban areas using machine learning. Water, 13(24), 3520. https://doi.org/10.3390/w13243520

- Li, B., Gao, Y., Wan, R., Dai, X., & Zhang, Y. (2016). Comparison of random forests and other statistical methods for the prediction of lake water level: a case study of the poyang lake in china. Hydrology Research, 47(S1), 69-83. https://doi.org/10.2166/nh.2016.264

- Li, X., Yan, D., Wang, K., Weng, B., Qin, T., & Liu, S. (2019). Flood risk assessment of global watersheds based on multiple machine learning models. Water, 11(8), 1654. https://doi.org/10.3390/w11081654

- Liu, J., Wang, J., Xiong, J., Cheng, W., Sun, H., Yong, Z., … & Wang, N. (2021). Hybrid models incorporating bivariate statistics and machine learning methods for flash flood susceptibility assessment based on remote sensing datasets. Remote Sensing, 13(23), 4945. https://doi.org/10.3390/rs13234945

- Lü, X., Zhong, M., & Zha, D. (2022). Runoff forecasting using machine-learning methods: case study in the middle reaches of xijiang river. Frontiers in Big Data, 4. https://doi.org/10.3389/fdata.2021.752406

- Mah, D. Y. S., Hin, L. S., & Putuhena, F. J. (2012). Investigative modelling of the flood bypass channel in kuching, sarawak, by assessing its impact on the inundations of kuching-batu kawa-bau expressway. Structure and Infrastructure Engineering, 8(7), 705-714. https://doi.org/10.1080/15732471003770167

- Mansour, M. (2020). A participatory approach for identification of micro flood zones in poorly developed urban areas. Academic Perspective Procedia, 3(2), 941-949. https://doi.org/10.33793/acperpro.03.02.32

- Mishra, V., Aaadhar, S., Shah, H. L., Kumar, R., Pattanaik, D. R., & Tiwari, A. D. (2018). The kerala flood of 2018: combined impact of extreme rainfall and reservoir storage.. https://doi.org/10.5194/hess-2018-480

- Mishra, V. and Shah, H. L. (2018). Hydroclimatological perspective of the kerala flood of 2018. Journal of the Geological Society of India, 92(5), 645-650. https://doi.org/10.1007/s12594-018-1079-3

- Mistry, S. and Parekh, F. (2022). Flood forecasting using artificial neural network. IOP Conference Series: Earth and Environmental Science, 1086(1), 012036. https://doi.org/10.1088/1755-1315/1086/1/012036

- Mosavi, A., Öztürk, P., & Chau, K. W. (2018). Flood prediction using machine learning models: literature review. Water, 10(11), 1536. https://doi.org/10.3390/w10111536

- Munawar, H. S., Hammad, A. W. A., & Waller, S. T. (2022). Remote sensing methods for flood prediction: a review. Sensors, 22(3), 960. https://doi.org/10.3390/s22030960

- Neeraj, S., Mannakkara, S., & Wilkinson, S. (2020). Build back better concepts for resilient recovery: a case study of india’s 2018 flood recovery. International Journal of Disaster Resilience in the Built Environment, 12(3), 280-294. https://doi.org/10.1108/ijdrbe-05-2020-0044

- Pant, R., Thacker, S., Alderson, D., & Barr, S. (2017). Critical infrastructure impact assessment due to flood exposure. Journal of Flood Risk Management, 11(1), 22-33. https://doi.org/10.1111/jfr3.12288

- Pham, B. T., Bui, D. T., Prakash, I., & Dholakia, M. B. (2017). Hybrid integration of multilayer perceptron neural networks and machine learning ensembles for landslide susceptibility assessment at himalayan area (india) using gis. Catena, 149, 52-63. https://doi.org/10.1016/j.catena.2016.09.007

- Qian, K., Mohamed, A., & Claudel, C. (2019). Physics informed data driven model for flood prediction: application of deep learning in prediction of urban flood development.. https://doi.org/10.48550/arxiv.1908.10312

- Rahman, M. T. A., Chen, N., Elbeltagi, A., Islam, M., Alam, M., Pourghasemi, H. R., … & Dewan, A. (2021). Application of stacking hybrid machine learning algorithms in delineating multi-type flooding in bangladesh. Journal of Environmental Management, 295, 113086. https://doi.org/10.1016/j.jenvman.2021.113086

- Rahman, M. M. and Davis, D. N. (2013). Addressing the class imbalance problem in medical datasets. International Journal of Machine Learning and Computing, 224-228. https://doi.org/10.7763/ijmlc.2013.v3.307

- Sampurno, J., Vallaeys, V., Ardianto, R., & Hanert, E. (2022). Integrated hydrodynamic and machine learning models for compound flooding prediction in a data-scarce estuarine delta.. https://doi.org/10.5194/npg-2021-36

- Sankaranarayanan, S., Prabhakar, M., Satish, S., Jain, P., Ramprasad, A., & Krishnan, A. (2019). Flood prediction based on weather parameters using deep learning. Journal of Water and Climate Change, 11(4), 1766-1783. https://doi.org/10.2166/wcc.2019.321

- Sarkar, D. and Mondal, P. (2019). Flood vulnerability mapping using frequency ratio (fr) model: a case study on kulik river basin, indo-bangladesh barind region. Applied Water Science, 10(1). https://doi.org/10.1007/s13201-019-1102-x

- Schumann, G., Bates, P., Horritt, M. S., Matgen, P., & Pappenberger, F. (2009). Progress in integration of remote sensing–derived flood extent and stage data and hydraulic models. Reviews of Geophysics, 47(4). https://doi.org/10.1029/2008rg000274

- Senan, C. P. C., Ajin, R. S., Danumah, J. H., Costache, R., Arabameri, A., Rajaneesh, A., … & Kuriakose, S. L. (2022). Flood vulnerability of a few areas in the foothills of the western ghats: a comparison of ahp and f-ahp models. Stochastic Environmental Research and Risk Assessment, 37(2), 527-556. https://doi.org/10.1007/s00477-022-02267-2

- Shabou, S., Ruin, I., Lutoff, C., Debionne, S., Anquetin, S., Creutin, J., … & Beaufils, X. (2017). Mobrisk: a model for assessing the exposure of road users to flash flood events. Natural Hazards and Earth System Sciences, 17(9), 1631-1651. https://doi.org/10.5194/nhess-17-1631-2017

- Shankar, M. A. and Bindu, C. (2021). Appraising the need for disaster mitigation in existing planning documents of municipal corporations of kerala in the event of past disasters. IOP Conference Series: Materials Science and Engineering, 1114(1), 012039. https://doi.org/10.1088/1757-899x/1114/1/012039

- Shiu, C., Liu, S. C., Fu, C., Dai, A., & Sun, Y. (2012). How much do precipitation extremes change in a warming climate?. Geophysical Research Letters, 39(17), n/a-n/a. https://doi.org/10.1029/2012gl052762

- Singh, P. K., Sinha, V. S. P., Vijhani, A., & Pahuja, N. (2018). Vulnerability assessment of urban road network from urban flood. International Journal of Disaster Risk Reduction, 28, 237-250. https://doi.org/10.1016/j.ijdrr.2018.03.017

- Tabarestani, E. S. and Afzalimehr, H. (2021). Artificial neural network and multi-criteria decision-making models for flood simulation in gis: mazandaran province, iran. Stochastic Environmental Research and Risk Assessment, 35(12), 2439-2457. https://doi.org/10.1007/s00477-021-01997-z

- Tabbussum, R. and Dar, A. Q. (2021). Performance evaluation of artificial intelligence paradigms—artificial neural networks, fuzzy logic, and adaptive neuro-fuzzy inference system for flood prediction. Environmental Science and Pollution Research, 28(20), 25265-25282. https://doi.org/10.1007/s11356-021-12410-1

- Tamiru, H. and Wagari, M. (2021). Machine-learning and hec-ras integrated models for flood inundation mapping in baro river basin, ethiopia. Modeling Earth Systems and Environment, 8(2), 2291-2303. https://doi.org/10.1007/s40808-021-01175-8

- Tayfur, G., Singh, V. P., Moramarco, T., & Barbetta, S. (2018). Flood hydrograph prediction using machine learning methods. Water, 10(8), 968. https://doi.org/10.3390/w10080968

- Tehrany, M. S., Pradhan, B., & Jebur, M. N. (2015). Flood susceptibility analysis and its verification using a novel ensemble support vector machine and frequency ratio method. Stochastic Environmental Research and Risk Assessment, 29(4), 1149-1165. https://doi.org/10.1007/s00477-015-1021-9

- Tehrany, M. S., Pradhan, B., & Jebur, M. N. (2014). Flood susceptibility mapping using a novel ensemble weights-of-evidence and support vector machine models in gis. Journal of Hydrology, 512, 332-343. https://doi.org/10.1016/j.jhydrol.2014.03.008

- Thakur, P. K., Ranjan, R., Singh, S., Dhote, P. R., Sharma, V., Srivastav, V., … & Chouksey, A. (2020). Synergistic use of remote sensing, gis and hydrological models for study of august 2018 kerala floods. The International Archives of the Photogrammetry, Remote Sensing and Spatial Information Sciences, XLIII-B3-2020, 1263-1270. https://doi.org/10.5194/isprs-archives-xliii-b3-2020-1263-2020

- Varughese, A. and Purushothaman, C. (2021). Climate change and public health in india: the 2018 kerala floods. World Medical &Amp; Health Policy, 13(1), 16-35. https://doi.org/10.1002/wmh3.429

- Veerakumaran, G. and Santhi, S. (2019). Impact assessment of kerala flood 2018 on agriculture of farmers in edathua panchayat, kuttanad taluk of alappuzha district. Shanlax International Journal of Economics, 7(4), 24-28. https://doi.org/10.34293/economics.v7i4.618

- Vishnu, C., Sajinkumar, K., Oommen, T., Coffman, R. A., Thrivikramji, K. P., Rani, V. R., … & Keerthy, S. (2019). Satellite-based assessment of the august 2018 flood in parts of kerala, india. Geomatics, Natural Hazards and Risk, 10(1), 758-767. https://doi.org/10.1080/19475705.2018.1543212

- V.L., G. (2021). The impact of flood in 2018 on the socio-economic conditions of the dairy farmers in pariyaram panchayat of thrissur district of kerala. Journal of Veterinary and Animal Sciences, 52(3). https://doi.org/10.51966/jvas.2021.52.3.272-276

- Wang, S., Wang, H., Wu, Z., & Zhou, Y. (2021). Using multi-factor analysis to predict urban flood depth based on naive bayes. Water, 13(4), 432. https://doi.org/10.3390/w13040432

- Ward, P. J., Eisner, S., Flörke, M., Dettinger, M. D., & Kummu, M. (2014). Annual flood sensitivities to el niño–southern oscillation at the global scale. Hydrology and Earth System Sciences, 18(1), 47-66. https://doi.org/10.5194/hess-18-47-2014

- Wu, Z., Zhou, Y., & Wang, S. (2020). Real-time prediction of the water accumulation process of urban stormy accumulation points based on deep learning. IEEE Access, 8, 151938-151951. https://doi.org/10.1109/access.2020.3017277

- Xie, X. and Shen, J. (2021). Waterlogging resistance evaluation index and photosynthesis characteristics selection: using machine learning methods to judge poplar’s waterlogging resistance. Mathematics, 9(13), 1542. https://doi.org/10.3390/math9131542

- Yang, S. and Chang, L. (2020). Regional inundation forecasting using machine learning techniques with the internet of things. Water, 12(6), 1578. https://doi.org/10.3390/w12061578

- Yin, J., Yu, D., Yin, Z., Lü, M., & He, Q. (2016). Evaluating the impact and risk of pluvial flash flood on intra-urban road network: a case study in the city center of shanghai, china. Journal of Hydrology, 537, 138-145. https://doi.org/10.1016/j.jhydrol.2016.03.037

- Zhang, D., Peng, Q., Wang, D., Yang, T., Sorooshian, S., Liu, X., … & Zhuang, J. (2018). Modeling and simulating of reservoir operation using the artificial neural network, support vector regression, deep learning algorithm. Journal of Hydrology, 565, 720-736. https://doi.org/10.1016/j.jhydrol.2018.08.050